# Comparison of LED Datasheets

## 1. Overview & Application

| Feature | JT-M5744OBSC5744 (Jointec) | HD-4MX00 (Hongzheng Optoelectronic) | NSSM237BT (Nichia) |  |
| --- | --- | --- | --- | --- |
| Application | Outdoor P8-P20 full color LED display | General RGB LED (indoor/outdoor) | General RGB LED (indoor/outdoor) |  |
| Features | Durable, moisture-proof, water-proof, anti-UV | High brightness, colored diffused lens | High brightness, black package |  |
| Package Type | Molding SMD | SMD | SMD, heat-resistant polymer |  |

## 2. Electrical & Optical Characteristics

### Absolute Maximum Ratings

| Parameter | JT-M5744OBSC5744 (Ta=25°C) | HD-4MX00 (Ta=25°C) | NSSM237BT (Ta=25°C) |  |
| --- | --- | --- | --- | --- |
| Forward Current (IF) | Blue/Green: 25 mA, Red: 35 mA | Blue: 10 mA, Green/Red: 20 mA | Blue/Green: 35 mA, Red: 50 mA |  |
| Pulse Current (IFP) | Blue/Green: 70 mA, Red: 150 mA | Blue: 55 mA, Green/Red: 50 mA | Blue/Green: 110 mA, Red: 150 mA |  |
| Reverse Voltage (VR) | 10 V | 25 V | 5 V |  |
| Power Dissipation | 80 mW (per die), 100 mW (total) | 55 mW (Blue), 30 mW (Green/Red) | 113 mW (Blue), 101 mW (Green), 125 mW (Red), 270 mW (total) |  |
| Operating Temp. | -40°C to +80°C | -35°C to +85°C | -40°C to +85°C |  |
| Storage Temp. | -40°C to +80°C | -40°C to +100°C | -40°C to +100°C |  |

### Optical Performance (Ta=25°C)

| Parameter | JT-M5744OBSC5744 | HD-4MX00 | NSSM237BT |  |
| --- | --- | --- | --- | --- |
| Luminous Intensity | Blue: 500-1500 mcd, Green: 4500-7500 mcd, Red: 500-1500 mcd | Blue: 460-660 mcd, Green: 2300-3400 mcd, Red: 1000-1450 mcd | Blue: 1.05-1.32 cd, Green: 5.90-7.44 cd, Red: 2.10-2.64 cd |  |
| Wavelength | Blue: 465-475 nm, Green: 521-531 nm, Red: 620-625 nm | Blue: 466-475 nm, Green: 526-535 nm, Red: 620-625 nm | Blue: 465 nm, Green: 520 nm, Red: 625 nm |  |
| Forward Voltage (VF) | Blue: 2.2-3.6 V, Green: 2.0-3.6 V, Red: 2.2-3.6 V | Blue: 2.5-3.1 V, Green: 2.5-3.1 V, Red: 1.8-2.4 V | Blue: 2.65-3.25 V, Green: 2.20-2.90 V, Red: 1.80-2.50 V |  |

## 3. Reliability & Environmental Testing

| Test | JT-M5744OBSC5744 | HD-4MX00 | NSSM237BT |
| --- | --- | --- | --- |
| Temperature/Humidity | 85°C/85% RH, 168 hours | 85°C/85% RH, 168 hours | 60°C/90% RH, 500 hours |
| Thermal Shock | -65°C to 150°C, 1000 cycles | -65°C to 150°C, 200 cycles | -40°C to 100°C, 100 cycles |
| Reflow Soldering | 260°C, 3 times | 255°C, 3 times | 260°C, 2 times |
| UV Resistance | 63°C/50% RH, 500 hours | 0.89 W/m² @ 340 nm, 14 cycles | Not specified |

## 4. Packaging & Handling

| Feature | JT-M5744OBSC5744 | HD-4MX00 | NSSM237BT |
| --- | --- | --- | --- |
| Packaging | 2500/tray | 3000/reel | 2000/reel |
| Moisture Sensitivity | MSL 3 | MSL 3 | MSL 4 |
| Baking Requirements | 60°C±5°C, 24 hours (if exposed) | 70°C±5°C, 12-48 hours (if exposed) | 65°C±5°C, 24 hours (if exposed) |

# Key Takeaways

1. Best for Outdoor Use: JT-M5744OBSC5744 (Jointec)

- Why? It is specifically designed for outdoor LED displays (P8-P20), with features like waterproofing, moisture-proofing, and anti-UV properties.

- Strengths: Durable, high brightness, and excellent angle consistency.

- Ideal for: Outdoor signage, billboards, and environments exposed to harsh weather.

2. Best for High Brightness: HD-4MX00 (Hongzheng Optoelectronic)

- Why? It offers the highest luminous intensity for green and red LEDs, making it ideal for applications where brightness is critical.

- Strengths: Colored diffused lens for better light distribution, high reliability in temperature/humidity tests.

- Ideal for: Indoor/outdoor displays where brightness and color consistency are priorities.

3. Best for Reliability and Precision: NSSM237BT (Nichia)

- Why? Nichia provides detailed specifications, strict testing standards, and comprehensive failure criteria. It is designed for high reliability and precision.

- Strengths: High thermal stability, strict ESD protection, and detailed guidelines for handling, soldering, and storage.

- Ideal for: High-end applications, such as professional displays, automotive lighting (if compliant), and environments where reliability is non-negotiable.

4. Electrical and Optical Performance

- JT-M5744OBSC5744: Lower forward current and power dissipation, making it energy-efficient for outdoor use.

- HD-4MX00: Higher luminous intensity, especially for green and red, but lower maximum forward current.

- NSSM237BT: Higher forward current and power dissipation, suitable for high-power applications.

5. Handling and Storage

- Moisture Sensitivity: All three LEDs require careful handling and storage, but NSSM237BT (MSL 4) is the most sensitive to moisture and requires stricter baking conditions.

- ESD Protection: All three require ESD protection, but Nichia provides the most detailed guidelines.

6. Soldering and Thermal Management

- Reflow Soldering: JT-M5744OBSC5744 and HD-4MX00 allow for more reflow cycles compared to NSSM237BT.

- Thermal Management: NSSM237BT provides the most comprehensive thermal management guidelines, ensuring longevity in high-temperature environments.

7. Environmental Testing

- JT-M5744OBSC5744 and HD-4MX00 have undergone rigorous environmental testing, including UV resistance and salt spray tests, making them more robust for outdoor applications.

- NSSM237BT focuses on thermal shock and moisture resistance, ensuring reliability in varying conditions.

8. Packaging and Availability

- JT-M5744OBSC5744: Packaged in trays (2500 units).

- HD-4MX00: Packaged in reels (3000 units).

- NSSM237BT: Packaged in reels (2000 units), with strict moisture-proof packaging.

## Summary Table

| Use Case | Best Choice | Reason |
| --- | --- | --- |
| Outdoor LED Displays | JT-M5744OBSC5744 (Jointec) | Waterproof, anti-UV, durable |
| High Brightness Applications | HD-4MX00 (Hongzheng) | Highest luminous intensity for green and red |
| High Reliability Applications | NSSM237BT (Nichia) | Strict testing, detailed specs, thermal stability |
| General Indoor/Outdoor Use | HD-4MX00 or NSSM237BT | Balanced performance and reliability |